Arboricultural Comments (Development applications):

Date:					19/01/11

Comments by:		J. Brewin (Arboricultural Officer – South)

Comments to:				Linda Griffiths (Senior Planning Officer – Major Developments)

CDC Reference:			10/01492/REM

Site Address:	KM6 Development site, Middleton Stoney Road, Bicester

Applicant / Agent (highlight one):	Bovis Homes Ltd

Applicants name/address (if different):		-

Statutory Protection:			 N/A

T.E.M.P.O guidance notes completed:	 No

Date of site visit:	Not taken.  Comments based upon submitted information and desktop research.

Trees / Proposal (brief description):	REM matters to Outline Planning Application 06/00967/OUT concerning landscaping proposals and tree planting.

Significant comments:

The constraints within the site leave little room for adequate tree planting leaving only the boundaries and the main access route into the site to provide the remotest opportunities for the establishment and development of trees which, should they reach maturity may provide adequate aesthetic, architectural and environmental benefits. It is disappointing to see a development such as this with no adequate internal green space to accommodate even a small percentage of significant tree coverage.

Due to required sight lines, desired views, access and adjacent plots planting quantities to the south and south-west boundaries are restricted however a slight increase in quantity of species such as acer campestre and betula pendula should be considered.

Additional plantings may be reduced during establishment by natural failure, vandalism and arboricultural thinning operations leaving the more established and better developed specimens

to mature. The northern boundary, consisting predominantly of quercus robur and alnus glutinosa appears to be adequate in quantity and requires no additional plantings.

Due to the boundary plantings being in close proximity to paved surfacing and footpaths all planting pit specifications must consider and include appropriate root barrier systems to reduce the risks of future ground disturbances which may create trip hazards and additional repair costs.

The main access into the site has been afforded 6 No trees consisting of 2 No Carpinus betulus ‘Frans Fontaine’ and 4 No alnus glutinosa ‘Imperialis’. The 2 No carpinus betulus are considered to be good choices for this location complimenting the boundary alders in form and with suitable form and mature width of canopy (4.0m) for their planting location.

The ‘Imperialis’, although a good choice for urban planting due to a tolerance of pollution, may develop a wider canopy capable of reaching 8.0m in spread. Due to the spatial restrictions associated with the access road and to promote continuity of planting into the site, I would prefer to replace the ‘Imperialis’ with c.b ‘Frans Fontaine’.

Once agreed these 4 No trees proposed for the access road will require suitable planting pits to ensure maximum chances of successful establishment and reduced risks of future ground disturbance as they reach maturity.

Specially designed planting pits such as the ‘Root Cell’ system provided by Greenleaf, allows for a continuous underground tree trench capable of accommodating adequate root growth for two or more trees thus improving the growing medium for the trees whilst reducing the risks of damage by surface tree roots. Although requiring engineering construction prior to road surfacing, this system is reputedly fast to install and cost-effective in planting pit construction.

Two separate sections of ‘Root Cell’ will be sufficient if installed the complete length of the two separate parking areas in which the four trees are proposed adjacent to the access drive.

Standard tree planting pits will not be sufficient in providing the trees with the growth space or nutrients required for successful establishment or mature growth.

Recommendations:

a) Consider increasing tree planting quantities of field maple and birch along the south west boundary line.

b) Amend the 4 No alnus glutinosa ‘Imperialis’ to 4 No Carpinus betulus ‘Frans Fontaine’

c) Install two separate tree trenches of ‘Root Cell’ to accommodate the planting of the above four trees.