- From: Jon Brewin 
Sent: 28 November 2014 11:31
To: Linda Griffiths
Cc: Tim Screen
Subject: 14/01682/REM - Land South West Of Bicester Adjoining Oxford Road And Middleton Stoney Road Bicester

Linda,

After reviewing the submitted information I would like to provide the following points:

         Plots 101, 102 and 103 must be relocated outside of the potential crown spread of the trees located within the agreed habitat corridor. The current proposed position of these plots will place the trees and the objectives of the habitat corridor under increasing threat culminating in the inevitable removal of the trees (birch, pine and rowan) within 12 – 15 years.  As this planting scheme is already agreed, the plots must be relocated and set back by a minimum of 5 – 7.0m.

         Two of the three trees identified within the boundary of Plot 74 should be relocated into the public realm outside of the boundary and placed within hard surface planting pits at either end of the adjacent visitor parking bay.

         The group of trees proposed for the open space area to the south-east of Plot 48 should be reduced in number and replaced with a single feature tree planting of 1 No Carpinus betula ‘Frans Fontaine’. This will provide a significant aesthetic feature in a prominent location within the development.

         The planting scheme within the central LAP to the front of Plots 166 – 168 should be amended to show 4 No Tilia mongolica replacing the 4 No proposed Prunus sunset boulevard. The limes will provide a level of height and aesthetic value to the street scene unmatched by the cherries.

         The proposed Acer campestre ‘Elsrijk’ for the Secondary Street are generally acceptable and suitable for the street scene however, the 5 No maples proposed to the frontages of Plots 180 – 183 will out-grow that location creating nuisance issues for the adjacent residents. These five trees should be replaced with 5 No Betula pendula’fastigiata’ whose upright, narrow form and less denser canopies will be more suitable.

         The proposed trees to the North-west of Plots 97, 98 and 120 should be removed from the scheme due to growth conflict with the agreed planting within the ‘Greenway’.

         The planting scheme for LAP to the North-West of the site and adjacent to the ‘Green Corridor’ should be amended to show 2 No Quercus robur parallel to the Green Corridor and 1 No Prunus sunset boulevard located within the eastern corner of the LAP. The remaining 4 No proposed trees within the LAP should be removed from the scheme to improve natural surveillance.

         The ‘visitor parking bays’ proposed for the Side Streets 2, 3 and 4 should have 2 No liquidamber styraciflua planted in hard surface pits at either end of each bay. This will place additional trees within the public streetscene. Any adjacent tree to these parking bays and located within narrow verge space or private garden area should be removed from the scheme to avoid growth conflict with the 6 No Liquidambers

         Plot 7: Remove 2 of the 3 proposed trees and retain the single Crataegus prunifolia.

         Plot 8: Retail 1 No single Liquidamber spp.

         Mews No 1: The 2 No Sorbus ‘Sheerwater Seedling’ should be relocated within the gardens of Plots 23 & 24 instead of the narrow verge areas proposed which fail to provide sufficient space for growth. This will allow for the retention of a single ‘feature tree’ to be planted in a structured cell planting pit located on the narrow verge strip between parking bays 37 & 38.

         Mews No 2: Remove the verge tree proposed  adjacent to Plot 46 should be removed as the space allocated is entirely inadequate in accommodating a tree. The four remaining Sorbus trees should be planted within structured cell planting pits.

         Mews No 3: 2 No trees proposed for verge planting outside the boundaries of Plots 141 & 155 should be removed due to inadequate space. The remaining 5 No Prunus should remain to be planted within structured cell planting pits.

         Mews No 4: No comment

         Specifications regarding all structured cell planting pits should be amended to specify appropriate volume of soil and cells to allow the individual tree to reach maximum development.

Regards

Jon Brewin 
Arboricultural Officer