SITE VISIT REPORT

Site Address:  Kingsmere (SW Bicester) Development, Bicester         Date: 26 January 2018

Developer: Countryside Properties Ltd (CP)

Planning Reference Number (s): 06/00967/OUT

Date of Visit: 25 January 2018	Meeting with: Richard Emery Senior Project Manager (Infrastructure) of Countryside Properties (CP) and Matt Stoddard of Gavin Jones

Photographs Taken?: Y/N	Yes (defective tree and concrete surround to gas chambers)

Summary of Visit:

On a request for a final completion inspection of the landscaped area prior to the issue of Certificates of Final Completion, by Paul Gibbs of David Jarvis Associates, as per his letter of 23 February 2015, Tim Green, CDC Landscape Officer and Tim Screen CDC Landscape Architect carried out inspections of the landscaping to ascertain if the CDC–adoptable areas had reached final completion. The area in questions is:

Middleton Stoney Road /Shakespeare Drive Roundabout (SL5): 09/00174/REM

Summary of Inspections and Recommendations

Final completion was not reached because of the following defects.

General comment:

There is much surface stone under the hedgerows; CP to remove off site and lay bark mulch

Shrub areas are bare of mulch and CP to lay mulch throughout. Treat moss growth in grassed areas and overseed with amenity grass where required. Flag poles including gas monitor pole to be made easier by spraying out 300 mm dia. circles and laying gravel. This will help the contractor’s operative the see the base of the poles while mowing and prevent damage to CP property.

Area West of Roundabout

- CP will retain, and maintain on good order, the timber post and rail fence the highway access haulage road for reasons of safety.

- Replace damaged Sorbus aucuparia (Sa) tree (main leader has removed low down the stem and it will never achieve the stature and symmetrical canopy of this species). Located as the last tree at the end of the row.

- The leaning tree (Sa) is to be supported with three timber stakes and tree straps to straighten it to vertical (avoiding major tree roots when installing the stakes)

- CP to check the site location of the high voltage cable (sign indicates this) and prove a drawing showing its location

- Remove hessian cloth from around the trunk of a silver birch tree.

- Three silver birch trees are growing in the hedgerow (very close to the paving and this may cause structural damage if left to grow on). Remove trees and replace them with 3 Crataegus monogyna

- The concrete surround to gas chamber is cracked and liable to be damaged with grass mowing. A new reinforced concrete surround is required with a specification acceptable to the Gas Company and CDC.

- The adoption red line boundary adjacent to (it is assumed either Taylor Wimpy (TW) or OCC highway) path behind the laurel hedge adjacent to house frontages need to be clarified by CP. There is a wide verge of poor stony soil between the laurel hedge and kerb edge of the path. Ideally the red line should go along the outer edge of the kerb and OCC/TW should agree to this red line amendment to allow the verge to become part of the land to be transferred to CDC. CP is to investigate with the objective of amending the red line boundary for the title deeds

Area East of Roundabout

- The ‘New Homes’ sign will be remove off site and infill planted shrubs Brachyglottis 'Sunshine' by CP when no  longer required

- Double manhole edge exposed and required topsoiling and grading to surrounding finished levels. Overseed with amenity grass seed.

- Remove ‘air pots’ material from around the truck of two trees on the shrub area

- The adoption red line boundary adjacent to (it is assumed OCC highway) path behind the laurel hedge does not concur with the HMS Land Registry (pink) land ownership plans, and should ideally align against the back of the kerb. CP is to investigate with the objective of amending the red line boundary and a

Conclusion

Upon rectification of defects identified above CP are to formally write to Linda Griffiths, Planning Case Officer, to confirm this stage and request final completion. CDC officers will then inspect the area and provide a report in accordance with the agreed protocol and 106 Agreement. CDC officers agreed that CP will continue to maintain the original meadow areas in the eastern area as close mown grass until legal transfer to CDC.