SITE VISIT REPORT

Site Address:  Kingsmere (SW Bicester) Development, Bicester         Date: 01.03.2018

Developer: Countryside Properties (UK) Ltd (CP)

Planning Reference Number (s): 06/00967/OUT

Date of Visit: 28.02.2018	Meeting and site inspection with: Emily Davis, Landscape Architect, David Jarvis Assoc.(DJA) Richard Emery, Senior Project Manager of Countryside Properties Ltd (CP) and Matt Studart of Gavin Jones

Photographs Taken?: Y/N	No

Summary of Visit:

Further to DJA’s Paul Gibbs request for a formal Practical Completion (PC) inspection (letter of 19.02.2018), Tim Green, CDC Landscape Officer (South) and Tim Screen (TS) CDC Landscape Architect joined above attendees to carry out an inspection of Enhanced LEAP. Practical Completion is confirmed.

RE:  Enhanced LEAP 12/00116/REM Defects – Recommendations

- CP to ensure the redline adoption boundary to the play area and associated open space accords with the land Registry Title Deeds.

- CP to encourage OCC to adopt the 3 m wide macadam-surfaced access to the school and amend the redline boundary accordingly.

- A local resident identified flooding issues (photographic evidence produced). CP to monitor the situation and install drainage to the affected areas within 3 months from the date of this report (CP to provide construction details).

- A general comment concerning the edges of the wetpour safer surface: the edge has come away from the kerb edge (e.g. roundabout circulation area) CP to re-bond the wetpour in accordance with Playdale’s instructions

- A couple of long bolt threads on the fence were noted as child finger traps, including 3 hexagon nut ends that need to be twisted off. All threads are to be cut and filed flushed with the lock nut and painted with grey rust-inhibiting paint.

- Cracking evident in macadam surface near southern entrance – overband with bitumen.

Conclusion

CDC would not wish to adopt the 3 m wide access to the school because of maintenance commitments and associated expense. CP and CDC to make enquiries with OCC to encourage them to adopt the route. CP to tackle the flooding issue.

The RoSPA play area safety inspection report expires on 25.07.2018, and CP will be required to provide an updated report.