SITE VISIT REPORT

Site Address:  Kingsmere (SW Bicester) Development, Bicester         Date: 29th June 2018

Developer: Countryside Properties (UK) Ltd (CP)

Planning Reference Number (s): 06/00967/OUT

Date of Visit: 27th June 2018	Site inspection attended by:Tim Green CDC Landscape Officer (South), Iain Osenton (IO) CDC Arboricultural Officer and Tim Screen (TS) CDC Landscape Architect

Photographs Taken?: Y/N	Yes – TS and IO

Summary of Visit:

Further to DJA’s Paul Gibb’s email request for final completion inspections of the areas indicated below the above persons inspected this areas

SL4 Howes Wood: 11/00380/REM

SL21 Signalised Junction: 12/00127/REM

SL21 Greenway: 12//00127/REM

SL13 and 20 Perimeter Road (Part): 09/01531/REM

Summary of Inspections and Recommendations

The SL13 and 20 Perimeter Road (Part) has achieved final completion and the FC certificate will be issued in due course. For the other areas, there are outstanding defects for CP to rectify.

SL4 Howes Wood  Drawing no. DFD/BIC/L14 Rev L

- An ivy-clad dead tree near to the pedestrian circulation has not been pollarded (opposite house no. 35)

- Heras fencing and stored fence panels are still evident (eastern end near play area)

- A remnant of an old fence has been exposed by the clearance of vegetation on the edge of the woodland (SW edge)

- Damaged fence on boundary of calcareous grassland

SL21 Signalised Junction  Drawing no. 2226/005 Rev C

- CP to side up hedge on both sides in the northern section, and remove arisings off site.

- Replace 10 dead/dying transplants in boundary hedgerow to the school.

- Replacements are to have rabbit guards supported by canes.

The two Alder trees in the aforementioned hedgerow are suffering from drought – CP are to water to field capacity during the current drought.

- A newly planting tree within the red line boundary, and adjacent to the A41, has died - CP to replace.

- Replenish mulch to tree rings and under hedgerows to 50 mm depth.

- Apply selective herbicide to grass areas to eradicate weeds.

- Grass arisings have not been collected.

- Two dying Ash where noted. Because of ash dieback these trees – CP to replace with two native birch, Betula pendula.

SL21 Greenway: 12//00127/REM

- Replacement tree is dead opposite house no. 78 (adjacent to hedgerow). No tree   support or mulch in evidence – CP to replace

- Re-stake leaning tree opposite house no. 50 near hedgerow.

- Apply 50 mm depth of mulch to the base of hedgerow and trees.

- Apply selective herbicide the grass to eradicate weed growth.

- Tree tie broken on tree opposite house no. 34 – CP to replace

- Silver Birches are suffering from the drought – CP to water to capacity

- Cleanly cut away dead stems of Silver Birch opposite house no. 24

- Dying tree near house no. 8 to be replaced by CP

SL13 and 20 Perimeter Road (Part): 09/01531/REM

- The recently planted transplants require watering to field capacity during the current drought – signs of stress were noted.

- CP are to clarify whether the advertising hoarding is to remain on site.

Conclusion

The items listed above shall be addressed by CP. Where replacement tree and hedge material is required it is good industry practice to plant during the next planting season, unless containerised stock is sourced and planted, and watered regularly to field capacity.